****************************************************************************

PROGRAM: Fluent_injection

This program writes a file containing injection points for FLUENT.

The user specifies the number of points desired, as well as the desired

plane (x-y, x-z, or y-z).  The program writes the appropriately formatted

file with the injection points for FLUENT.

To obtain the location of the injection points within a circle, the program first

defines equally spaced points within a square having the same side length as the

desired diameter of the circle.  The points are examined, row by row and column by

column, and only those points falling within the diameter of the circle are printed.

The coordinates are offset by the origin (i.e., (0,0) assumed at the center of the circle) and

axial location of the desired injection plane.  The offset coordinates are printed to the

user-prescribed file, and an additional file with coordinates centered at the origin are

printed to another file with ‘*_origin.dat’ as the file name.  The total number of injection

points that fall within the circle may not be exactly the same as the user-specified number

of points.

The required input parameters are contained in an input file that is named by the user.

The parameters in the input file are defined below.

Cliff Ho, (505) 844-2384, ckho@sandia.gov

May 19, 2008

Input File for Fluent_injection.f90 (this example is for the Calgon reactor)

10000          ! npts: number of injection points desired

xz          ! plane: desired plane for injection points (xy, xz, yz)

3.4036      ! coord: axial coordinate of the plane (m)

0.3048      ! dia: diameter of injection circle (m)

-1.3716     ! coord1: 1st coordinate of injection-plane centroid (m)

0.          ! coord2: 2nd coordinate of injection-plane centroid (m)

0.          ! u: u-velocity component of injected particle (m/s)

0.          ! v: v-velocity component of injected particle (m/s)

0.          ! w: w-velocity component of injected particle (m/s)

1e-6        ! dp: particle diameter (m)

20.         ! tp: temperature of injected particle (C)

0.          ! mp: mass flow rate of injected particle (kg/s)

10000.inj           ! outfile: name of desired output file

! These are the input parameters for the Fluent_injection.f90 code

! Cliff Ho, ckho@sandia.gov, (505) 844-2384, 5/19/08

Sandia is a multiprogram laboratory operated by Sandia Corporation, a Lockheed Martin Company for the United States Department of Energy’s National Nuclear Security Administration under contract DE-AC04-94AL85000.